Provided by Chesapeake Employers Insurance

Maryland’s heat stress standard establishes minimum requirements for employers to protect employees from heat-related illnesses caused by heat stress in the workplace. The standard took effect on Sept. 20, 2024, and is enforced by the Maryland Occupational Safety and Health (MOSH).

Notably, the standard requires covered employers to develop heat-related illness prevention and management plans, monitor the heat index, develop high heat and emergency response procedures, provide employees with drinking water and access to shade, allow employees to acclimatize to the heat and train employees on heat stress.

The standard applies to Maryland employers when their employees work in areas where the heat index equals or exceeds 80 degrees Fahrenheit.

Excluded from the standard are:

- Emergency operations (emergency responders and utility providers responding to emergencies should review for specific applicability);

- Incidental exposures (when an employee performs work activities for less than 15 consecutive minutes per hour); and

- Buildings, structures and vehicles that have a mechanical ventilation system or fan that maintains the heat index below 80 degrees Fahrenheit.

The following checklist outlines the steps employers should take to comply with the standard.

- Section 1: Heat Index Monitoring

Covered employers must monitor the heat index where employees perform work using a prescribed method.

| Heat Index Monitoring Requirements | Yes | No |
| --- | --- | --- |
| Does the employer monitor the heat index through the shift in areas where employees perform work using one of the following methods? - Direct measurement of temperature and humidity; - Use of local weather data reported by the National Weather Service or other recognized source; or - Use of the National Institute for Occupational Safety and Health’s (NIOSH) Heat Safety Tool. NOTE: Direct measures of temperature and humidity must be used when employees work in buildings and structures that do not have a mechanical ventilation system. | ☐ | ☐ |

- Section 2: Written Heat-related Illness Prevention and Management Plan

Employers must develop, implement and maintain an effective heat-related illness prevention and management plan. These plans must be written and made available to employees and MOSH upon request.

| Written Heat-related Illness Prevention and Management Plan | Yes | No |
| --- | --- | --- |
| Does the written plan include information on how sufficient drinking water will be provided to employees? Note: See Section 5 for more details. | ☐ | ☐ |
| Does the written plan include procedures for how employees will be provided opportunities and encouragement to stay hydrated by drinking water? | ☐ | ☐ |
| Does the written plan include information on how employees can recognize the symptoms of heat-related illness, including heat exhaustion and heat stroke? | ☐ | ☐ |
| Does the written plan include information on how employees should respond to confirmed or suspected cases of heat-related illness, including heat exhaustion and heat stroke? | ☐ | ☐ |
| Does the written plan include procedures for ensuring employees have sufficient time and space to rest in shaded or climate-controlled areas to cool off? | ☐ | ☐ |
| Does the written plan outline how the employer will implement rest break schedules as necessary? | ☐ | ☐ |
| Does the written plan include information on how the employer will consider environmental conditions, workload, required clothing, personal protective equipment (PPE), and alternative cooling and control measures when determining rest break schedules? | ☐ | ☐ |
| Does the written plan include information on how the employer will encourage employees to take rest breaks as needed to prevent heat-related illness? | ☐ | ☐ |
| Does the written plan include information on the use and maintenance of alternative cooling and control measures used to manage heat? Note: “Alternative cooling and control measures” are defined under the standard as engineering, work-practice, administrative or other controls to manage heat, including job rotation, mechanical ventilation systems, misting equipment, cooling vests, air-cooled or water-cooled garments, and access to recreational water. | ☐ | ☐ |
| Does the written plan include information on how the employer provides employees and supervisors training on the hazards of heat exposure and the necessary steps to prevent heat-related illness? Note: See Section 8 for more detail. | ☐ | ☐ |
| Does the written plan include procedures for heat acclimatization? Note: See Section 3 for more details. | ☐ | ☐ |
| Does the written plan include procedures for high-heat conditions? Note: See Section 6 for more details. | ☐ | ☐ |
| Does the written plan include an emergency response plan for heat-related illnesses? Note: See Section 7 for more details. | ☐ | ☐ |

- Section 3: Acclimatization Procedures

- Under the standard, acclimatization refers to the body’s temporary adaptation to work in heat that occurs as a person is exposed to heat gradually over time. The standard prescribes specific acclimatization procedures that employers must follow.

| Acclimatization Procedures | Yes | No |
| --- | --- | --- |
| Does the employer provide acclimatization for up to 14 days for employees newly exposed to heat or employees returning from being away from work for seven or more consecutive days? | ☐ | ☐ |
| Does the employer monitor employees for signs of heat-related illness during acclimatization periods using one or more of the following methods of communication? - Phone; - Radio; - Buddy system; or - Another effective means of observation. | ☐ | ☐ |
| Do the employer’s acclimatization procedures comply with one of the following methods? - A schedule that gradually increases exposure time over a five- to 14-day period, with a maximum 20% increase in exposure time each day. - A schedule that uses the current NIOSH’s recommendations for acclimatization. - A schedule that uses a combination of gradual introduction and alternative cooling and control measures that acclimate an employee to the heat. | ☐ | ☐ |
| Are acclimatization schedules in writing and consider all of the following elements? - Acclimatized and unacclimatized employees; - The environmental conditions and anticipated workload; - The impact of required clothing and PPE on employees’ heat burden; - The personal risk factors that put an employee at a higher risk of heat-related illness; - Reacclimatizing employees as necessary, in accordance with the standard; and - The use of alternative cooling and control measures. | ☐ | ☐ |
| When developing employee acclimatization schedules, does the employer reference information provided by MOSH, OSHA, NIOSH or other recognized sources? | ☐ | ☐ |

- Section 4: Access to Shade

- Under the standard, employers must provide employees exposed to heat with access to shade.

| Access to Shade Requirements | Yes | No |
| --- | --- | --- |
| Does the employer provide shaded areas as close to the work area as practical? | ☐ | ☐ |
| Do the shaded areas provided to employees meet all of the following criteria? - Shaded areas must be outside, open and exposed to air on at least three sides; - Shaded areas must prevent contributing heat sources from reducing the effectiveness of the shade; - Shaded areas must be sufficient in size for the number of employees utilizing the shaded area; - Shaded areas must be arranged in a configuration that allows employees to sit in normal postures; and - Shaded areas must accommodate the removal and storage of PPE during periods of use. Notes: If creating outdoor shade is demonstrably infeasible or unsafe, the employer must implement alternative cooling and control measures that provide equivalent protection from heat. Indoor cooling with a mechanical ventilation system instead of outdoor shade is permitted, provided that the indoor space meets the requirements of the standard. | ☐ | ☐ |

- Section 5: Access to Drinking Water

- Under the standard, employers must provide employees with access to drinking water.

| Access to Drinking Water Requirements | Yes | No |
| --- | --- | --- |
| Does the employer provide drinking water to employees exposed to heat in excess of 80 degrees Fahrenheit? Notes: Employers must provide ready access to at least 32 ounces of drinking water per hour per employee. Drinking water must be potable, cool in temperature and provided free of charge. Employers are not required to provide the entire drinking supply to employees at the beginning of the employee’s shift, but employers must make drinking water available at all times when work is being performed in heat in excess of 80 degrees Fahrenheit. | ☐ | ☐ |
| Is drinking water positioned as close to the work area as practical? | ☐ | ☐ |

- Section 6: High-heat Procedures

- Under the heat stress standard, employers are required to implement high-heat procedures when the heat index reaches or exceeds 90 degrees Fahrenheit in the area where work is being performed.

| High-heat Procedures | Yes | No |
| --- | --- | --- |
| Does the employer implement high-heat procedures when the heat index reaches or exceeds 90 degrees Fahrenheit in the area where work is being performed? | ☐ | ☐ |
| Do high-heat procedures include a work and rest schedule to protect employees from heat-related illness that is adjusted for environmental conditions, workload, and impact of required clothing or PPE? | ☐ | ☐ |
| Does the employer provide minimum rest periods based on the heat index (10 minutes for every two hours worked at 90 degrees to 100 degrees Fahrenheit and 15 minutes for every hour worked above 100 degrees Fahrenheit)? Notes: Employers may provide a rest period as provided for in the current NIOSH recommendations for work and rest schedules to manage heat exposure. MOSH considers rest periods to be a part of the workday. An employer may coincide rest periods with a scheduled rest or meal period. All cool-down rest periods must be taken in shaded areas. Employers may not discourage employees from taking rest breaks as needed to prevent heat-related illnesses. | ☐ | ☐ |
| If utilizing alternative cooling and control measures in lieu of rest periods during high-heat procedures, does the employer do all of the following? - Ensure that such measures are readily available and accessible to employees at all times work is being performed; - Document such measures in writing; and - Ensure that such measures do not violate any requirements of the standard. | ☐ | ☐ |
| When high-heat procedures are in effect, does the employer monitor exposed employees for signs of heat-related illness with regular communication using any of the following methods? - Phone; - Radio; - Buddy system; or - Another effective means of observation. | ☐ | ☐ |
| Does the employer make high-heat procedures available in writing in a language and manner that all employees can understand? | ☐ | ☐ |

- Section 7: Emergency Response Procedure Requirements

- Under the standard, employers must implement emergency response procedures that address heat illness.

| Emergency Response Procedure Requirements | Yes | No |
| --- | --- | --- |
| Has the employer implemented emergency response procedures for heat illness that include all of the following elements? - Procedures ensuring effective and accessible means of communication at all times at the worksite to enable an employee to contact a supervisor or emergency medical services; - Procedures for responding to signs and symptoms of possible heat-related illness in employees; - Procedures for monitoring and providing care to employees who are exhibiting symptoms of heat-related illness; and - Procedures for contacting emergency medical services and, if necessary, transporting employees to a location accessible to emergency medical services. Note: MOSH expects employers to monitor employees and not leave employees who are exhibiting symptoms alone or send them home without offering first aid or providing medical treatment. Heat-related illness can mimic other illnesses, and prompt treatment is critical. | ☐ | ☐ |

- Section 8: Training Requirements

Employers must provide all employees and supervisors covered by the standard with heat stress training prior to an employee’s first exposure to heat. Employers must also provide retraining at least annually.

| Heat-related Illness Training Requirements | Yes | No |
| --- | --- | --- |
| Does the employer provide initial heat stress training to employees and supervisors prior to an employee’s first exposure to heat? | ☐ | ☐ |
| Does the employer retrain employees and supervisors on heat stress at least annually prior to exposure and immediately following any incidents in the workplace involving a suspected or confirmed heat-related illness? | ☐ | ☐ |
| Does the employer present training in a language and a manner that all employees and supervisors can understand? | ☐ | ☐ |
| Does the employer’s heat stress training include all of the following requirement elements? - Information on the work and environmental conditions that affect heat-related illnesses; - Information on the personal risk factors that affect heat-related illnesses; - Information explaining the concept, importance and methods for heat acclimatization; - Information on the importance of frequent consumption of water and rest breaks; - Information on the types of heat-related illnesses, the signs and symptoms of heat-related illnesses, and the appropriate first aid and emergency response measures in response to heat-related illnesses; - Information on the importance of and procedures for employees immediately reporting to the employer signs and symptoms of heat-related illness; and - Information on the employer’s procedures and the requirements for complying with Maryland’s heat stress standard. | ☐ | ☐ |
| Does the employer maintain training records for at least one year from the date on which the training occurred? | ☐ | ☐ |
| Do training records include the names of the persons trained, the date(s) of the training session(s) and a summary or outline of the content of the training session(s)? | ☐ | ☐ |
| Does the employer make training records available to MOSH upon request? | ☐ | ☐ |